Machines Involved

- Oasi

Default layout configuration

Layout initialization on new produced machines will be as reported in the following table:

Note: Model(layout) could be embeeded into machine sw or loaded from an USB stick.

| OASI (STD) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| MACHINE CODE | SW | COUNTRY | DEFAULT CONFIGURATION Model (Layout) Embedded or USB stick | DEFAULT CONFIGURATIONS | Recipes Sw Layout |
| 9A0002 OASI 400 ES7-R/I | V0.5.12 | Italy | [EMBEDDED] L02 Esp400 5S ITAESP | 1IN+2IN+2IN+1B2C |  |
| 9A0050 OASI 600 ES8-R/I | V0.5.12 | Italy | [EMBEDDED] L03 Esp600 6S ITAESP | 2IN+1IN+1IN+2IN1+1B2C | (To do) |
| 9A0003 OASI 400 ES6-R/F | V0.5.12 | France | L023_01Francia | 1IN+1IN+1IN+1IN+1B2C |  |
| 9A0051 OASI 600 ES6-R/F | V0.5.12 | France | L023_01Francia | 1IN+1IN+1IN+1IN+1B2C |  |
| F9A000400 OASI 400 ESB6-R/D GERMANIA | V0.5.12 | Germania | F9A000400_L024_01Germania | 1IN+1IN+1IN+2IN+1B2C |  |
| F9A005200_OASI 600 ESB6-R/D Germania | V0.5.12 | Germania | F9A005200_L024_01Germania | 1IN+1IN+1IN+2IN+1B2C |  |
| F9A0005 _L032 -Russia 400 ES6-R-RUS | V0.5.12 | Russia | F9A0005 _L030 -Russia | 1IN+1IN+2IN+1B2C |  |
| F9A0053_L031 -Russia 600 ESB7-R-RUS | V0.5.12 | Russia | F9A0053_L031 -Russia | 1IN+1IN+1IN+2IN+1B2C |  |
| F9A0006_L024_02 -Germania 400 ESB6-RD | V0.5.12 | Germania | F9A006_L024_02 -Germania | 1IN+1IN+1IN+2IN+1B2C |  |
| F9A0054_L024_02 - Germania 600 ESB6-RD | V0.5.12 | Germania | F9A006_L024_02 -Germania | 1IN+1IN+1IN+2IN+1B2C |  |
| F9A0055 L023_02_-Francia 600 ES7-RF | V0.5.12 | France | F9A0055 L023_03_-Francia 600 | 1IN+1IN+1IN+2IN+1B2C |  |
| 9A0008 OASI 400 ES7-RE-P _L030 Spagna 02 400 | V0.5.12 | Spain | 9A0008_OASI_400_ES7-RE-P_L30_1_0x15 | 1IN+2IN+1IN+1IN+1B2C |  |
| 9A0056 OASI 600 ES8-RE-P_L29_02_Spagna_600 | V0.5.12 | Spain | 9A0056_L29_1_0xC9 | 2IN+1IN+1IN+2IN+1B2C |  |
| 9A0007_L23_03-Francia 400 ES6-RF | V0.5.12 | France | 9A0007 L023_03Francia 400 | 1IN+1IN+1IN+1IN+1B2C |  |

Updated Modules

The release includes, for each firmware packages, a version of the following modules:

| SW VERSION: 0.05.12 |  |  |
| --- | --- | --- |
| Program Module | Revision Number | Date |
| Oasi Actuator Board | ### V0.05.09 [3DED3FB7] | 01/06/21 |
| OASI Capacitve Keyboard | ### V.71 [B4FE] | 23/05/19 |
| Updater | ### 18 [5D6D] | 12/07/13 |

values  inside square brackets [x] are CRC that could be checked in Autotest -

Languages: Italian, German, French, English, Dutch, Spanish, Portoguese, Turchish, Russian

(custom language could be added using tool “Custom Text Translator OASI.xls”).

List of changes

0.05.12

- Reports of fake faults in the history events fixed #32939

0.05.11

- Changed cvd for empty airbreak.

- Changed time of heating in case of possible temperature sensor disconnected .

0.05.09

- Introduced a timed display reset to hide “blank display” issue when machine works with water tank kit. Timed reset could be enabled in Menu 1.17.3 selecting  " YES + Display Reset ".

- Fixed bug :beverage LED not lit during brewing if beverage belong to beverage set 2 or 3.

0.05.01

- -Payment system: added new Price Holding protocol “PHD remote” to allow products selection via app Breasy (implemented  Accept Selection 0x3C command, not implemented price alignement ). Just select "PHD Remote" to obtain that product X will be selled at X price .

- -. Fixed bug #7305 "Error 14 when machine starts up in a cold environment".(due to an HW limit that prevent reading temperature lower than 9°C). Now error signalling is delayed by SW allowing boiler to reach at least 11 °C .

0.04.01

- Released File “L023_01Francia.MOD” containing parameters for France OASI 400 and 600 machines;

- Released File “9A006_L24_2_0xFA_Germania.MOD” and “9A0054_L24_2_0xFA_Germania.MOD”

- Released File “9A0005_L030_01_0x18_Russia400.MOD” and “9A0053_L031_01_0x04_Russia600.MOD”

- Released File “L023_01Francia.MOD” containing parameters for France OASI 400 and 600 machines;

- Added WATER LEAKAGE ERROR (code 82). Machine will go in Out of service in case of water leakage from air-break;

- Added Doser Blocked Error (code 47). Machine will disable coffee product if doser blocked.

- Added mangement for Sugar as soluble (for model with no sugar on the door);

- Added menu 2.14 to enable price refund in case of failed sells;

- Improved USB key management. Fixed problems with multiple read/write operations.

- Autotest: added test for 5 keys keyboard;

- Autotest: added test to check coffee Brewing Unit;

- Total beverage water showed in recipe menu;

0.03.71

- Released File “L023_01Francia.MOD” containing parameters for France OASI 400 and 600 machines;

- Electromechanical Product counter: counts every beverage (not just coffee as in previous versions);

- Fixed items numbering in Error LOG menu;

- Improved boiler emptying cycle : reduced water level into Airbreak and moved backward nozzle arm  at cycle end;

- Improved Error Log on USB – added error code description;

- Added a boiler refill before beverages if machine not used for more than 10 minutes;

0.03.66

- Added a parameter to adjust dispensing position for noozle arm. In System Management 1.15.2 it is possible to set a delay to improve nozzle centering on the cup;

- Default prices are set to 650.00;

- Changed default value for System Management 1.9 (EV to be used for hot water products “W” in recipes) from 0 (Solenoid Kit) to 2;

- Now also selections without coffee shows the delivery screen with no delay;

- Autotest Airbreak: now airbreak test works also on “400 model”;

- Autotest Grinder: added control on coffee dispenser now when dispenser is full of coffee powder the message "POSSIBLE ERROR” appears on display

- -  improved exit from programming mode to avoid occasional operation of loads;

- -  Autotest: lowered execution time, improved some procedure

Programming Instructions (System update at boot)

SW Update Package

(USB Stick for update at boot)

For the "Update at boot" procedure the USB stick used must be formatted as FAT32, and in the root folder there must be present the following file:

SAE_0000.s19

Procedure:

Plug it in the machine before power on, Press button P1 on CPU board and keep pressed, Power ON machine . Release P1 when CPU beeps.

CPU search for s19 file on USB and starts programming (no indication on the screen).

The programming will last five minutes.

At the end of the programming machine beeps three times and restart (if necessary power board and keyboard will be automatically updated).

If some problem will raise during the programming, it will be possible to repeat the procedure.